McMag2000 Utility Program and USB Driver Installation Instructions

The McMag2000 uses the FTDI Chip USB drivers.

Plug the USB cable into the PC and into the McMag2000.

The computer should recognize that a new device has been plugged in and try to install the drivers for it.  If the installation fails (does not install drivers), the driver files probably need to be downloaded.

You can verify if the driver files are installed by going to the Device Manager and looking at the Ports (COM & LPT) and see if the USB Serial Port (COMxx) is listed.

If it isn’t, then there should be an unrecognized device with a yellow exclamation character next to it.

The driver can be downloaded from here:

https://www.ftdichip.com/Drivers/CDM/CDM%20v2.12.28%20WHQL%20Certified.zip

The files need to be Unzipped.

Right-mouse click on the device with the yellow exclamation point and click on Update Driver.

Click on Browse my Computer for Driver Software.

Click on the Browse button and located the folder of the extracted files.

Click Ok.

Click Next.

The USB Driver file should be installed.

You may need to repeat these steps again in order for the Virtual Comport to be installed.  It uses the same files as the USB Driver.

After it is done, you should see the USB Serial Port (COMxx) listed in the Ports (COM & LPT) list.

The McMag2000 Utility Program was written by Eric Mikkelsen.

There are no installation files.

The program and support files can be copied or unzipped into a folder and run from there.

Here is a link to the program and files:

https://mccrometer.box.com/s/waq1s723nt2jn7y8tffp2sg86fn689uu

The structure of the files are as follows:

Download the zipped file from the link above and extract the files.

Right-mouse click on the program and click on Run as administrator.

The program should execute.

Make sure that drop-down for the port shows the USB Serial Port (COMxx) that was created when the USB and Virtual Comports was installed.